EMBEDDED SYSTEMS LAB REPORT

Controlling a Servo Motor Using a Potentiometer

Project Type: Embedded Systems / Arduino Practical
Microcontroller: Arduino Uno
Analog Input: A1
Servo Control: D9
Primary Concepts: analogRead(), map(), Servo library, myservo.write()

1. Project Overview

This practical demonstrates analog input processing and servo motor control using an Arduino Uno. A 10k potentiometer was connected to analog input pin A1 and used to provide a variable voltage to the Arduino. The potentiometer reading was converted into a servo angle between 0° and 180°, allowing the angular position of an SG90 micro servo motor to be controlled by rotating the potentiometer.

The exercise provided practical experience with reading analog inputs, converting sensor-like input values into a usable control range and interfacing an actuator with a microcontroller using the Arduino Servo library.

2. Aim

To control the angular position of a servo motor using a potentiometer as an analog input and demonstrate how analogRead(), map() and the Servo library can be used to read a variable voltage, convert it into a servo angle and control the position of the motor.

3. Apparatus and Components

| Component | Quantity | Purpose |
| --- | --- | --- |
| Arduino Uno microcontroller board | 1 | Reads the potentiometer input and controls the servo motor |
| USB cable | 1 | Provides the programming/data connection between the computer and Arduino |
| Breadboard | 1 | Provides a temporary, solderless circuit platform |
| 10k potentiometer | 1 | Provides a variable analog voltage input |
| SG90 micro servo motor | 1 | Converts the control signal into a variable angular position |
| Jumper wires | As required | Provides electrical connections between components |
| Computer with Arduino IDE | 1 | Used to write, verify and upload the sketch |

4. Circuit Configuration

The potentiometer was connected as a variable voltage divider. Its two outer terminals were connected to the 5V and GND rails, while the middle terminal (wiper) was connected to analog input A1 on the Arduino Uno. Rotating the potentiometer therefore changed the voltage supplied to A1.

The SG90 servo motor was connected to digital pin 9 for its control signal. The servo's power wire was connected to the 5V rail and its ground wire was connected to the common GND rail.

The connections used were:

| Arduino Pin | Component | Connection |
| --- | --- | --- |
| A1 | Potentiometer | Wiper → A1 |
| 5V | Potentiometer / Servo | Power connection |
| GND | Potentiometer / Servo | Ground connection |
| D9 | SG90 Servo | Signal wire → D9 |

Figure 1. Simplified circuit connection: the potentiometer provides a variable analog input to A1, while digital pin 9 provides the servo control signal.

Arduino Uno

5V ─────────────┐

│

[10kΩ]

│

├──── Wiper ──── A1

│

GND ─────────────┘

D9 ───────────────────── Servo Signal

5V ───────────────────── Servo Power

GND ───────────────────── Servo Ground

5. Procedure

➢ The 10k potentiometer was placed on the breadboard. Its two outer terminals were connected to the 5V and GND rails, while the middle terminal (wiper) was connected to Arduino analog pin A1.

➢ The SG90 micro servo motor was connected to the breadboard. Its signal wire was connected to digital pin 9, its power wire was connected to the 5V rail and its ground wire was connected to the GND rail.

➢ The Arduino Uno was connected to the computer using the USB cable.

➢ In the Arduino IDE, the Servo library was included and a Servo object named myservo was created.

➢ The servo was associated with digital pin 9 using myservo.attach(9) inside setup().

➢ In loop(), the potentiometer value was read from analog pin A1 using analogRead().

➢ The resulting analog value ranging from 0 to 1023 was mapped to a servo angle range of 0° to 180° using the map() function.

➢ The mapped value was sent to the servo using myservo.write(val).

➢ A 15 ms delay was included between position updates to allow stable and controlled servo movement.

➢ The sketch was verified and uploaded to the Arduino Uno.

➢ The potentiometer was rotated by hand while the servo horn was observed. The servo responded to changes in potentiometer position by adjusting its angular position.

6. Software Implementation

Potentiometer Input and Servo Control

The program uses the Arduino Servo library to control the SG90 servo motor. The servo is attached to digital pin 9 during setup().

During each iteration of loop(), the potentiometer value is read from analog input A1 using analogRead(). The Arduino converts the variable input voltage into a numerical value between 0 and 1023. The map() function then converts this range into a corresponding servo angle between 0° and 180°.

The resulting angle is sent to the servo using myservo.write(val). A short 15 ms delay is used between updates to provide stable movement.

#include <Servo.h>

Servo myservo;

int val;

void setup(){

myservo.attach(9);

}

void loop(){

val = analogRead(1);

val = map(val, 0, 1023, 0, 180);

myservo.write(val);

delay(15);

}

The overall signal-processing sequence can therefore be represented as:

Potentiometer

↓

Analog voltage

↓

analogRead()

↓

Value: 0–1023

↓

map()

↓

Angle: 0–180°

↓

myservo.write()

↓

Servo position

7. Results and Observations

The servo motor was successfully controlled using the potentiometer as an analog input. Rotating the potentiometer caused the servo horn to change its angular position, demonstrating that the servo response followed changes in the potentiometer input.

The potentiometer's analog reading was successfully converted from the range 0–1023 into a corresponding servo angle between 0° and 180°. The observed servo movement matched the programmed relationship between the potentiometer position and the desired servo angle.

8. Conclusion

The aim of the practical was successfully achieved by using an Arduino Uno to control an SG90 servo motor through a 10k potentiometer. The potentiometer provided a variable analog input to pin A1, which was read using analogRead() and converted from the 0–1023 input range to a 0–180° servo angle using map().

The mapped value was then sent to the servo using the Servo library and myservo.write(), allowing the servo's angular position to respond to the potentiometer position. The practical demonstrated the interaction between analog input processing and actuator control, providing a foundation for more advanced embedded-systems applications involving sensors, motors and real-time control.